- Positive impact of Monsoonal Ground Water Rise:

Influence of 1 m sustainable monsoonal ground water level in reducing stress on Irrigation Pumps in bore wells (Equivalent Power Saving in KWHrs) in Kuppam constituency/ KADA Region is 22,930 KWHrs and Chitoor Distrit is 2,09,075KWHrs.

Methodology:-

Data :-

As per GEC Seasonal ground water utilization for KADA  Region, Draft Q =5880 Ham /year or 1.87 m3/sec and for  Chittoor District , Draft Q =53793 Ham /year or 17.05 m3/sec  ,  Time conversion Hr-Sec , t= 3600 Sec, Pumping period T=7 Hr, Functional Days F=100 Days,  Desity =1000 Kg/m3 , Gravity constant =9.81 m/s2 , Water level change H=1 m

Monsoonal Ground Water Rise & reduced Stress on Irrigation Pumps

| Period | Monsoon Rise in Ground Water Levels | Reduced Stress on Irrigation Pumps (Equivalent Power Saving in (KWHrs) |
| --- | --- | --- |
| Previous Water Year (May2024-Nov-2024) | 1.68 m (19.67-17.99) | 38,520 |
| Present Water Year (May-2025to Nov-2025) | 5.04 m (20.30-15.26) | 1,15,560 |
| Improvement (Compare with last year) | 3.36 m (5.04 -1.68) | 77,040 |

- Decadal /Last 10 years Premonsoon Average Ground water level in KADA is about 18.31 m bgl considred as Reference water level.

- Previous Water Year  (May2024-Nov-2024) Monsoon Rise of Ground Water Levels in KADA is 1.68 m (from 19.67 m to 17.99m).

- It is 9.17 % of Reference Water level 18.31 m bgl.

- Present Water Year (May-2025to Nov-2025) Monsoon Rise of Ground Water Levels in KADA is 5.04 m (from 20.30 m to 15.26 m).

- It is 27.52 % of Reference Water level 18.31 m bgl.

- Increase in Monsoonal Rise in Ground Water Levels about 3.36 m (from 1.68 to 5.04 m).

- It is 18.35 % of Reference Water level 18.31 m bgl.

- Ground Water Level Zones:

Areawise about 8%area increased in Shallow Water zone (3-8 m Zone)  and about 8% area reduced in Deep Water Zone (>20m Zone)

| Region | Period | Zone Area( in %) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | <3m | 3-8m | 8-20m | >20m |
| KADA Region | Premonsoon /May-2025 | 0 | 7.7 | 53.8 | 38.5 |
|  | Post Monsoon /November 2024 | 7.7 | 7.7 | 53.8 | 30.8 |
|  | Post Monsoon / November 2025 | 7.7 | 15.4 | 53.8 | 23.1 |